# ĐÃ CHUYỂN: dùng file canonical cho server có Nginx sẵn:

#   deploy/nginx/host/tracking.david.io.vn.conf

#   + deploy/SETUP_EXISTING_NGINX_HTTPS.md

#

# File này giữ làm bản đầy đủ (có limit_req) nếu bạn tự gộp zone vào http{} — dễ trùng tên với site khác.

#

# DNS A: tracking.david.io.vn → 160.191.50.186 (hoặc IP server hiện tại)

#

# 1) Sao chép deploy/nginx/conf.d/00-tracking-limits.conf → /etc/nginx/conf.d/

# 2) Sao chép file này → /etc/nginx/sites-available/tracking.david.io.vn

# 3) sudo ln -sf /etc/nginx/sites-available/tracking.david.io.vn /etc/nginx/sites-enabled/

# 4) sudo nginx -t && sudo systemctl reload nginx

#

# Backend chỉ lộ trên loopback — Nginx proxy tới 127.0.0.1:6510

#

# --- Hardening (chống flood / slowloris nhẹ, giảm tải backend) ---

# - limit_req / limit_conn: cần file conf.d 00-tracking-limits.conf

# - Chỉnh rate= và burst= nếu traffic hợp lệ bị 429 (tăng nhẹ) hoặc muốn siết hơn

server {

listen 80;

listen [::]:80;

server_name tracking.david.io.vn;

# Body nhỏ — API tracking JSON (khớp backend ~256kb, dư một chút)

client_max_body_size 512k;

# Chống client treo khi gửi header/body chậm (slowloris kiểu cũ)

client_body_timeout 12s;

client_header_timeout 12s;

send_timeout 20s;

# 429 khi vượt hạn

limit_req_status 429;

limit_conn_status 429;

limit_req zone=tracking_req burst=100 nodelay;

limit_conn tracking_conn 50;

location / {

proxy_pass http://127.0.0.1:6510;

proxy_http_version 1.1;

proxy_set_header Host $host;

proxy_set_header X-Real-IP $remote_addr;

proxy_set_header X-Forwarded-For $proxy_add_x_forwarded_for;

proxy_set_header X-Forwarded-Proto $scheme;

# Timeout proxy — tránh worker Nginx bị giữ lâu bởi backend chậm / tấn công

proxy_connect_timeout 10s;

proxy_send_timeout 45s;

proxy_read_timeout 60s;

# Giới hạn buffer upstream (tránh response bất thường chiếm RAM)

proxy_buffering on;

proxy_buffers 16 16k;

proxy_busy_buffers_size 64k;

}

}

# --- HTTPS (bật sau certbot): copy cùng các chỉ thị client_*, limit_*, proxy_* vào server 443 ---

#

# server {

#     listen 443 ssl http2;

#     listen [::]:443 ssl http2;

#     server_name tracking.david.io.vn;

#

#     ssl_certificate     /etc/letsencrypt/live/tracking.david.io.vn/fullchain.pem;

#     ssl_certificate_key /etc/letsencrypt/live/tracking.david.io.vn/privkey.pem;

#

#     client_max_body_size 512k;

#     client_body_timeout 12s;

#     client_header_timeout 12s;

#     send_timeout 20s;

#     limit_req_status 429;

#     limit_conn_status 429;

#     limit_req zone=tracking_req burst=100 nodelay;

#     limit_conn tracking_conn 50;

#

#     location / {

#         proxy_pass http://127.0.0.1:6510;

#         proxy_http_version 1.1;

#         proxy_set_header Host $host;

#         proxy_set_header X-Real-IP $remote_addr;

#         proxy_set_header X-Forwarded-For $proxy_add_x_forwarded_for;

#         proxy_set_header X-Forwarded-Proto $scheme;

#         proxy_connect_timeout 10s;

#         proxy_send_timeout 45s;

#         proxy_read_timeout 60s;

#         proxy_buffering on;

#         proxy_buffers 16 16k;

#         proxy_busy_buffers_size 64k;

#     }

# }

#

# --- Nếu đứng sau Cloudflare: dùng IP thật của khách cho limit (bỏ comment + chỉnh CF IP ranges) ---

# real_ip_header CF-Connecting-IP;

# set_real_ip_from 173.245.48.0/20;

# set_real_ip_from 103.21.244.0/22;

# ... (xem https://www.cloudflare.com/ips/)